ECA0403 – ANALOG CIRCUITS

DESIGN ASSIGNMENT

# DESIGN, ANALYZE AND OPTIMIZE THE LOW-FREQUENCY RESPONSE

# OF A COMMON-EMITTER BJT AMPLIFIER

Complete Design, LTspice Simulation, Verification and Optimization Report

Course: Analog Circuits Simulation Tool: LTspice Date: 02 September 2026

Student Name: B. Kavithri

Register No.: 192319064

Department: B.E Biomedical engineering

Engineering Faculty: Dr. N. Shyamala

# Table of Contents

- Executive Summary

- Problem Statement and Design Requirements

- Source Materials and Evidence Basis

- Circuit Topology and Component Selection

- Theoretical Analysis and Design Calculations

- LTspice Implementation and Simulation Setup

- DC Operating-Point Analysis

- AC Frequency-Response Analysis – Original Design

- Theoretical versus Simulated Verification

- Design Modification and Optimization to ≈50 Hz

- Transient-Time-Domain Verification

- Original versus Optimized Comparison

- Engineering Interpretation, Trade-offs and Practical Considerations

- Rubric-Wise Requirement Mapping

- Limitations and Evidence Gaps

- Conclusion and Recommendations

- Student Reflection

- References and Source Acknowledgement

Appendix A – Final Component and Simulation Checklist Appendix B – Key LTspice Measurements

Note: Page numbers should be updated in Word after final formatting if a dynamic table of contents is inserted.

# Executive Summary

This report presents the design, analysis, LTspice simulation, verification and optimization of a single-stage voltage-divider-biased common-emitter (CE) BJT amplifier. The assignment specifies a 12 V supply, RC = 2.2 kΩ, RE = 1 kΩ, RL = 10 kΩ, RS = 600 Ω, transistor current gain β = 100, a mid-band voltage gain of approximately 20 V/V, and a lower cutoff frequency of approximately 100 Hz.

The completed LTspice design uses R1 = 1.5 kΩ and R2 = 270 Ω as the voltage-divider bias network, C1 = 10 µF, C2 = 10 µF and CE = 68 µF for the original design, and CE = 158 µF for the optimized design. The transistor is represented by the custom QNPN model visible in the supplied schematic, with BF = 100, VAF = 100, IS = 2e-15 A, CJE = 10 pF, CJC = 5 pF, TF = 1 ns and TR = 10 ns.

The LTspice operating-point result gives VB ≈ 1.828 V, VE ≈ 1.130 V, VC ≈ 9.536 V and VCE ≈ 8.406 V, placing the transistor in the forward-active region. The original AC response gives a mid-band gain of 25.9067 dB (≈19.74 V/V), fL = 99.871 Hz and fH = 4.060903 MHz. After increasing CE to 158 µF, the measured lower cutoff becomes 48.9564 Hz, an error of only 2.09% from the 50 Hz target, while the upper cutoff remains approximately 4.061 MHz. Transient analysis independently gives Vout(pp) = 394.915 mV for Vin(pp) = 20 mV, corresponding to a measured voltage gain of approximately 19.75 V/V.

# Problem Statement and Design Requirements

The supplied assignment brief identifies the task as “Design and Simulation of a Common-Emitter BJT Amplifier Using LTspice.” It requires a single-stage CE amplifier using a voltage-divider bias network and explicitly specifies the electrical targets and analysis tasks. The assignment brief is the controlling source for the parameter set used in this report.	filecite turn3file0 L30-L65

| Parameter / Requirement | Specification / Task | Status |
| --- | --- | --- |
| Supply voltage, VCC | 12 V | Satisfied |
| Collector resistor, RC | 2.2 kΩ | Satisfied |
| Emitter resistor, RE | 1 kΩ | Satisfied |
| Load resistance, RL | 10 kΩ | Satisfied |
| Source resistance, RS | 600 Ω | Satisfied |
| Transistor current gain, β | 100 | Satisfied by QNPN model BF=100 |
| Mid-band voltage gain | Approximately 20 V/V | Satisfied: ≈19.74– 19.75 V/V |
| Lower cutoff frequency | Approximately 100 Hz | Satisfied: 99.871 Hz |
| AC sweep | Suitable range; example 10 Hz–1 MHz | Implemented as 10 Hz–100 MHz |
| Required extracted quantities | fL, mid-band gain, fH, bandwidth | Completed |
| Verification | Theory vs LTspice; gain and fL error | Completed |
| Optimization | Change only capacitor values; target fL ≈50 Hz | Completed with CE = 158 µF |

The assignment also states that commercially available standard component values should be used wherever possible and that the final LTspice plot should clearly show low-frequency, mid-band and high-frequency regions.

filecite turn3file0 L66-L81

# Source Materials and Evidence Basis

The report is based on the following supplied materials and evidence:

Assignment brief – Untitled document.pdf: Primary source for specifications, tasks, expected outputs and 100-mark rubric.	filecite turn3file0 L11-L81

Laboratory manual – LAB MANUAL ECA04 ELECTRONIC CTRCUIT S PCB DESIGN new.docx: Supporting laboratory guidance for CE amplifier gain, dB conversion and bandwidth calculation. It gives Av = Vo/Vi, gain(dB) = 20 log10(Av), and BW = f2 − f1.	filecite turn2file2 L127-L143

LTspice schematic screenshots: Evidence of the implemented component values, wiring, source settings, transistor model and simulation directives.

LTspice operating-point screenshot: Evidence of simulated node voltages and device currents.

LTspice AC sweep screenshots with cursors: Evidence for mid-band gain, fL and fH.

LTspice transient screenshots with cursors: Evidence for input/output waveforms and output peak-to-peak measurement.

Important source-reconciliation note: a previously stored supporting report in the file library contains a different CE amplifier parameter set (RC = 4 kΩ, RL = 2 kΩ, R1 = 82 kΩ and R2 = 15 kΩ). Those values do not match the supplied ECA0403 assignment brief or the final LTspice screenshots used here, so they were not used to override the actual assignment design. The present report follows the assignment brief and the final student LTspice evidence.

# Circuit Topology and Component Selection

The implemented amplifier is a single-stage common-emitter BJT voltage amplifier. The base is biased using a voltage-divider formed by R1 and R2. The input source is coupled to the base through C1, the collector is connected to VCC through RC, the emitter is connected to ground through RE, and CE bypasses RE for AC operation. The collector output is coupled through C2 to the load RL. RS models the source resistance.

Figure 1. Original LTspice CE amplifier schematic used for the 100 Hz design. Source: supplied LTspice screenshot.

| Component | Value / Setting | Function |
| --- | --- | --- |
| VCC | 12 V DC | Collector supply |
| V2 | SINE(0 10m 1k), AC 1 | 10 mV peak, 1 kHz input; AC analysis excitation = 1 |
| Q1 | QNPN, BF = 100 | Amplifying device |
| R1 | 1.5 kΩ | Upper voltage-divider bias resistor |
| R2 | 270 Ω | Lower voltage-divider bias resistor |
| RC | 2.2 kΩ | Collector load resistor |
| RE | 1 kΩ | Emitter resistor; DC stabilization |
| RS | 600 Ω | Source resistance |
| RL | 10 kΩ | External load |
| C1 | 10 µF | Input coupling capacitor |
| C2 | 10 µF | Output coupling capacitor |
| CE | 68 µF | Emitter bypass capacitor, original design |

The custom transistor model is explicitly visible in the schematic as .model QNPN NPN(BF=100 VAF=100 IS=2e-15 CJE=10p CJC=5p TF=1n TR=10n). This makes the stated β = 100 directly represented in the simulation model and also supplies the junction and transit parameters that influence the high-frequency response.

# Theoretical Analysis and Design Calculations

## Voltage-Divider Bias

For the selected R1 = 1.5 kΩ and R2 = 270 Ω:

VTH = VCC × R2/(R1 + R2) = 12 × 270/(1500 + 270) = 1.8305 V

RTH = R1 || R2 = 228.81 Ω

Using β = 100, RE = 1 kΩ and an assumed VBE = 0.70 V, IB = (VTH − VBE) / [RTH + (β + 1)RE] = 11.17 µA. IC = βIB = 1.117 mA; IE = (β + 1)IB = 1.128 mA.

VB = VTH − IBRTH = 1.828 V

VE = IERE = 1.128 V

VC = VCC − ICRC = 9.543 V VCE = VC − VE = 8.415 V

## Small-Signal Parameters and Mid-Band Gain

At mid-band, the coupling and bypass capacitors are treated as approximately short circuits. Using VT ≈ 25 mV:

gm = IC/VT = 0.04467 S

rπ = β/gm = 2238.6 Ω R1 || R2 = 228.81 Ω

Rin(base) = rπ || R1 || R2 = 207.59 Ω RC || RL = 1803.28 Ω

Stage gain ≈ −gm(RC || RL) = -80.56 V/V

Including the 600 Ω source resistance, the overall theoretical gain is Av ≈ [−gm(RC || RL)] × Rin(base)/(RS + Rin(base)) = -20.71 V/V.

The negative sign indicates phase inversion. The magnitude is close to the assignment target of approximately 20 V/V.

## Low-Frequency Pole Estimates

The laboratory manual defines voltage gain as Av = Vo/Vi and gain in dB as 20 log10(Av), with bandwidth given by BW = f2 − f1.	filecite turn2file2 L136-L143	The three major low-frequency capacitors are considered separately for a first-order estimate.

| Pole / capacitor | Approximate expression | Calculated value |
| --- | --- | --- |
| Input coupling C1 = 10 µF | 1/[2πC1(RS + Rin)] | 19.71 Hz |
| Output coupling C2 = 10 µF | 1/[2πC2(RC + RL)] | 1.30 Hz |
| Emitter bypass CE = 68 µF | 1/[2πCE Rseen,E] | 79.41 Hz |
| Emitter bypass CE = 158 µF | 1/[2πCE Rseen,E] | 34.18 Hz |

These are individual first-order pole estimates, not the exact overall −3 dB cutoff of the complete amplifier. Because the three low-frequency networks interact, the final fL is obtained from the complete LTspice frequency-response curve. This distinction is important when comparing the analytical pole estimate with the measured composite cutoff.

# LTspice Implementation and Simulation Setup

The final LTspice schematic contains the complete signal path, source resistance, voltage-divider bias, transistor, collector and emitter resistors, coupling capacitors, emitter bypass capacitor and load. All return paths are connected to ground. The source is set to SINE(0 10m 1k), giving a 10 mV peak small-signal input at 1 kHz.

AC analysis directive used:

### .ac dec 100 10 100Meg

This performs a decade sweep with 100 points per decade from 10 Hz to 100 MHz. The wider-than-example range was used so that both low-frequency and high-frequency roll-off could be clearly observed.

Transient analysis directive used:

### .tran 0 10m 0 10u

At 1 kHz, a 10 ms run contains ten input cycles. The 10 µs maximum timestep provides adequate time resolution for the 1 kHz waveform and for measuring the peak-to-peak output.

# DC Operating-Point Analysis

Figure 2. LTspice operating-point result supplied for the biased amplifier.

The operating-point screenshot shows the following simulated node voltages and currents. The automatically generated node names were interpreted from the final schematic connectivity.

| Quantity | Theoretical estimate | LTspice | Percentage deviation |
| --- | --- | --- | --- |
| VB | 1.828 V | 1.82813 V | 0.010% |
| VE | 1.128 V | 1.13037 V | 0.214% |
| VC | 9.543 V | 9.53606 V | 0.073% |
| VCE | 8.415 V | 8.40569 V | 0.112% |
| IC | 1.117 mA | 1.11997 mA | 0.285% |
| IB | 11.17 µA | 10.3982 µA | — |

The collector voltage is substantially above the emitter voltage and VCE is about 8.41 V, indicating that the device is not close to saturation. The finite collector current also shows that the transistor is not in cutoff.

Therefore the operating point is consistent with the forward-active region required for approximately linear voltage amplification.

# AC Frequency-Response Analysis – Original Design

The original design uses CE = 68 µF. The AC response is displayed as magnitude and phase versus frequency. The flat mid-band region is approximately 25.91 dB. The −3 dB level relative to the mid-band value is:

Target −3 dB level = 25.906669 − 3 = 22.906669 dB.

Figure 3. Original-design AC sweep showing the mid-band cursor measurement at approximately 1.006 kHz.

The cursor in Figure 3 reads 1.00647 kHz and 25.906669 dB, providing the measured mid-band gain.

Figure 4. Original-design AC sweep with the cursor at the lower −3 dB crossing.

Figure 4 gives fL = 99.871159 Hz at 22.905988 dB, which is effectively the −3 dB point. This closely matches the 100 Hz design target.

Figure 5. Original-design AC sweep with the cursor at the upper −3 dB crossing.

Figure 5 gives fH = 4.060903 MHz at 22.929356 dB, which is within the expected cursor-point tolerance around the 22.9067 dB target.

| Original-design metric | Measured LTspice result | Interpretation |
| --- | --- | --- |
| Mid-band gain | 25.9067 dB | ≈19.74 V/V; close to 20 V/V target |
| Lower cutoff fL | 99.871159 Hz | ≈100 Hz target achieved |
| Upper cutoff fH | 4.060903 MHz | High-frequency roll-off captured |
| Bandwidth | ≈4.060803 MHz | fH − fL |

# Theoretical versus Simulated Verification

The theoretical gain calculation gives approximately 20.71 V/V when the source resistance, base-divider loading and 10 kΩ load are included. The measured LTspice gain is approximately 19.74 V/V. The difference is small and is expected because the theoretical calculation uses first-order small-signal assumptions, whereas LTspice uses the full nonlinear transistor model and exact network impedances.

Gain percentage error = |19.746 − 20.707| / 20.707 × 100 = 4.64%.

For the lower cutoff, the assignment target is approximately 100 Hz and LTspice gives 99.871159 Hz. The target-based deviation is 0.129%.

| Quantity | Theory / target | LTspice | Error / deviation |
| --- | --- | --- | --- |
| Mid-band gain magnitude | 20.71 V/V | 19.746 V/V | 4.64% |

| Mid-band gain | 26.32 dB | 25.9067 dB | consistent with magnitude |
| --- | --- | --- | --- |
| Lower cutoff fL | ≈100 Hz | 99.871 Hz | 0.129% |

No separate theoretical high-frequency cutoff calculation was supplied in the assignment materials. Therefore, the report does not invent a theoretical fH; the reported 4.060903 MHz value is explicitly presented as the measured LTspice result.

- Design Modification and Optimization to ≈50 Hz

The assignment requires the lower cutoff to be reduced to approximately 50 Hz by changing only capacitor values. The implemented modification changes CE from 68 µF to 158 µF while leaving VCC, resistors, transistor model, C1 and C2 unchanged.

Figure 6. Optimized LTspice schematic with CE = 158 µF.

Figure 7. Optimized AC sweep showing the lower cutoff at 48.956418 Hz.

At the optimized lower −3 dB crossing, the cursor reads fL = 48.956418 Hz and magnitude = 22.920763 dB. The 50 Hz target error is:

Percentage error = |50 − 48.956418|/50 × 100 = 2.09%.

This is a successful optimization: increasing the emitter bypass capacitance increases the bypass effect at lower frequencies and shifts the dominant low-frequency response toward lower frequency. The upper cutoff remains essentially unchanged because CE primarily controls the low-frequency emitter-bypass behavior.

Component-value compliance note: 158 µF is the value actually simulated and therefore the value reported here. The assignment asks for commercially available standard values wherever possible; the supplied evidence does not establish that 158 µF is the preferred commercially standard value. If strict standard-value compliance is required, 150 µF should be simulated separately and the measured result should be reported rather than assumed.

# Transient-Time-Domain Verification

A transient simulation was performed using the optimized design with the 10 mV peak, 1 kHz input. Both the input node V(n005) and output node V(n003) were plotted.

Figure 8. Transient response showing the small input waveform and amplified output waveform.

Figure 9. LTspice cursor evidence used to obtain the output minimum; combined with the measured output maximum.

From the supplied cursor measurements, the output maximum is +193.97144 mV and the output minimum is

−200.94379 mV. Thus:

Vout(pp) = 193.97144 − (−200.94379) = 394.91523 mV.

Vin(pp) = 2 × 10 mV = 20 mV.

Transient voltage gain = Vout(pp)/Vin(pp) = 394.91523/20 = 19.746 V/V.

The transient result therefore gives approximately 19.75 V/V, in excellent agreement with the AC small-signal result of approximately 19.74 V/V. The waveform also visibly shows the output inverted relative to the input, which is the expected phase reversal of a common-emitter amplifier.

The screenshots do not provide a dedicated cursor measurement of an exact 180° phase difference between the labelled input and output peaks. Therefore the report treats the phase result as a waveform-observed inversion rather than claiming a separately measured phase-cursor value.

# Original versus Optimized Comparison

| Metric | Original design | Optimized design | Change / observation |
| --- | --- | --- | --- |
| CE | 68 µF | 158 µF | Increased |
| C1 | 10 µF | 10 µF | Unchanged |
| C2 | 10 µF | 10 µF | Unchanged |
| Mid-band gain | ≈25.91 dB | ≈25.91 dB | Essentially unchanged |
| Voltage gain | ≈19.74 V/V | ≈19.75 V/V transient | Essentially unchanged |
| fL | 99.871 Hz | 48.956 Hz | Reduced by ≈50.96 Hz |
| fH | 4.060903 MHz | 4.060903 MHz | Essentially unchanged |
| Bandwidth | ≈4.060803 MHz | ≈4.060854 MHz | Slight numerical increase from lower fL |

The central engineering trade-off is that CE can be increased to extend the low-frequency response without changing the DC bias network. However, very large capacitance values increase physical size, cost and startup/transient time, while giving diminishing returns once the desired cutoff has already been reached.

# Engineering Interpretation, Trade-offs and Practical Considerations

## Reliability and bias stability

RE provides DC negative feedback and helps stabilize the operating point. The voltage-divider establishes the base bias independently of the AC source, allowing the transistor to remain in the active region for small-signal amplification.

## Component tolerance

The assignment requires consideration of practical component selection. Resistor and capacitor tolerances can shift the bias point and low-frequency poles. In particular, the cutoff frequency is sensitive to the actual capacitance and the resistance seen by each capacitor. A tolerance sweep would strengthen the design but was not part of the supplied simulation evidence.

## Power and safety

The amplifier operates from a 12 V DC supply. Practical hardware testing should use a current-limited regulated supply, verify capacitor polarity where polarized capacitors are used, and ensure a common ground reference for measurement equipment.

## Cost and manufacturability

The circuit uses a small number of ordinary resistors, capacitors and one BJT, making it low-cost and straightforward to prototype. The main practical concern is selecting a suitable standard capacitor value for the optimized design.

## Professional responsibility

All numerical simulation results in this report are taken from the supplied LTspice screenshots or calculations based on the stated circuit values. Where a measurement was not directly available, it has been explicitly identified as a limitation rather than replaced with an invented value.

# Rubric-Wise Requirement Mapping

The supplied assignment uses an eight-part 100-mark rubric covering problem understanding, circuit design, theoretical analysis, frequency response, LTspice simulation, theoretical-vs-simulated verification, optimization, and conclusion.	filecite turn3file0 L82-L145	filecite turn3file0 L161-L214

filecite turn3file0 L226-L345

| Rubric criterion | Evidence in this report | Assessment status |
| --- | --- | --- |
| 1. Problem Understanding & Design Requirements – 10 | Section 2; specification table | Directly addressed |
| 2. Circuit Design & Component Selection – 20 | Sections 4 and 5; schematic and component table | Directly addressed |
| 3. Theoretical Circuit Analysis – 15 | Section 5; bias, small-signal and pole calculations | Directly addressed |
| 4. Frequency-Response Analysis – 15 | Section 8; fL, gain, fH, bandwidth and interpretation | Directly addressed |
| 5. LTspice Simulation – 15 | Section 6; AC and transient settings plus screenshots | Directly addressed |
| 6. Theoretical vs Simulated Verification – 10 | Section 9; gain and fL deviations | Directly addressed |
| 7. Design Modification / Optimization – 10 | Section 10; CE changed and 48.956 Hz verified | Directly addressed |
| 8. Engineering Interpretation & Conclusion – 5 | Sections 13 and 16 | Directly addressed |

# Limitations and Evidence Gaps

- An exact theoretical high-frequency pole/fH calculation was not provided in the assignment materials; therefore fH is reported only from LTspice.

- The transient screenshots establish waveform inversion visually, but do not provide a dedicated exact phase-difference cursor measurement between Vin and Vout. The report therefore does not claim a separately measured 180° value.

- The optimized value CE = 158 µF is the value actually simulated. The supplied evidence does not establish whether 158 µF is the preferred commercially available standard value; a 150 µF verification run would be needed for strict standard-value compliance.

- No component-tolerance sweep, Monte Carlo analysis, or experimental hardware measurement was supplied. Consequently, tolerance-based statistical performance and hardware-versus-simulation comparison are not claimed.

- The automatic LTspice node names (for example V(n003) and V(n005)) are implementation-generated. Their identities were interpreted from the final schematic connectivity and plotting sequence.

# Conclusion and Recommendations

The required single-stage common-emitter BJT amplifier was successfully designed and simulated in LTspice using a 12 V supply, voltage-divider bias, 2.2 kΩ collector resistor, 1 kΩ emitter resistor, 10 kΩ load and 600 Ω source resistance. The selected bias resistors R1 = 1.5 kΩ and R2 = 270 Ω establish a stable operating point with LTspice values of approximately VB = 1.828 V, VE = 1.130 V, VC = 9.536 V and VCE = 8.406 V.

The original 68 µF emitter-bypass design achieves a measured mid-band gain of 25.9067 dB, equivalent to approximately 19.74 V/V, and a lower cutoff of 99.871 Hz. Thus, the two principal design targets—approximately 20 V/V gain and approximately 100 Hz lower cutoff—are both met. The upper cutoff is measured as 4.060903 MHz, giving a bandwidth of approximately 4.0608 MHz.

For optimization, only the emitter bypass capacitor was changed, from 68 µF to 158 µF. The measured lower cutoff moved to 48.9564 Hz, which is within 2.09% of the required 50 Hz. The upper cutoff and mid-band gain remain essentially unchanged, demonstrating that the capacitor modification successfully targets the low-frequency response without materially disturbing the mid- or high-frequency behavior.

Transient verification gives an output peak-to-peak voltage of 394.915 mV from a 20 mV peak-to-peak input, yielding a measured gain of 19.75 V/V. The output waveform is inverted relative to the input, confirming the expected common-emitter phase reversal.

Recommended final actions are: (1) retain the verified 68 µF design as the approximately 100 Hz reference design; (2) retain the 158 µF result only if that value is accepted as a practical capacitor choice; (3) otherwise simulate 150 µF and report its actual fL; and (4) if additional evaluation time is available, perform a resistor/capacitor tolerance sweep and a direct phase-cursor measurement to strengthen the validation evidence.

# Student Reflection

This assignment provided practical understanding of how a common-emitter BJT amplifier moves from a circuit diagram to a verified engineering design. The most important learning outcome was recognizing that correct DC biasing is essential before an AC amplifier result can be trusted. The voltage-divider network establishes the transistor operating point, while the source resistance and load resistance directly influence the overall voltage gain. This made the theoretical calculation more meaningful because the target gain cannot be obtained by considering the transistor alone.

Working with LTspice also showed the difference between a theoretical approximation and a complete circuit simulation. The hand calculation uses simplified small-signal parameters and assumptions such as VBE ≈ 0.70 V and VT ≈ 25 mV, whereas LTspice uses the specified transistor model and exact network impedances. The resulting gain difference was small, showing that the simplified model was useful for design while the simulator was necessary for final verification.

The frequency-response measurements were particularly useful because the lower cutoff was not treated as a number that had to be assumed. The cursor was used to locate the −3 dB point directly on the simulated response. The original circuit produced 99.871 Hz, which closely matched the 100 Hz requirement. Increasing the emitter bypass capacitor then reduced the cutoff to 48.956 Hz. This demonstrated the physical role of CE: increasing its capacitance lowers its reactance at a given frequency, allowing the emitter to be bypassed more effectively at lower frequencies.

If additional time and resources were available, the next improvement would be a tolerance analysis. Real capacitors and resistors do not have exact nominal values, so the actual lower cutoff and bias point would vary. A second improvement would be to compare the simulated circuit with a breadboard measurement using an oscilloscope. Finally, the optimized capacitor should be checked against commercially standard values so that the final hardware design is both electrically correct and easy to source.

# References and Source Acknowledgement

18. References and Source Acknowledgement

- ECA0403 Analog Circuits Design Assignment, “Design, Analyze and Optimize the Low-Frequency Response of a Common-Emitter BJT Amplifier,” supplied assignment PDF.

- ECA04 Electronic Circuits / PCB Design Laboratory Manual, CE amplifier experiment guidance and gain/bandwidth formulas.

- LTspice schematic, operating-point, AC sweep and transient-analysis screenshots supplied by the student in the conversation.

- LTspice custom QNPN model shown on the supplied final schematic: BF=100, VAF=100, IS=2e-15, CJE=10p, CJC=5p, TF=1n, TR=10n.

- AI-assisted drafting and numerical checking were used in preparing this report; all reported LTspice measurements were taken from the supplied simulation evidence.

# Appendix A – Final Component and Simulation Checklist

| Check | Required / final value | Verified |
| --- | --- | --- |
| VCC | 12 V | ✓ |
| R1 | 1.5 kΩ | ✓ |
| R2 | 270 Ω | ✓ |
| RC | 2.2 kΩ | ✓ |
| RE | 1 kΩ | ✓ |
| RS | 600 Ω | ✓ |
| RL | 10 kΩ | ✓ |
| C1 | 10 µF | ✓ |
| C2 | 10 µF | ✓ |
| CE – original | 68 µF | ✓ |
| CE – optimized | 158 µF | ✓ |
| Input source | SINE(0 10m 1k) | ✓ |
| AC directive | .ac dec 100 10 100Meg | ✓ |
| Transient directive | .tran 0 10m 0 10u | ✓ |
| Ground reference | Common LTspice ground | ✓ |
| Transistor model | QNPN, BF=100 | ✓ |

# Appendix B – Key LTspice Measurements

| Measurement | Recorded value | Evidence |
| --- | --- | --- |
| DC base voltage VB | 1.82813 V | Operating-point screenshot |
| DC emitter voltage VE | 1.13037 V | Operating-point screenshot |
| DC collector voltage VC | 9.53606 V | Operating-point screenshot |
| DC VCE | 8.40569 V | Calculated from VC − VE |
| Original mid-band gain | 25.906669 dB | AC cursor screenshot |
| Original gain magnitude | ≈19.74 V/V | 20log conversion |
| Original fL | 99.871159 Hz | AC cursor screenshot |
| Original fH | 4.060903 MHz | AC cursor screenshot |
| Original bandwidth | ≈4.060803 MHz | fH − fL |
| Optimized CE | 158 µF | Optimized schematic screenshot |
| Optimized fL | 48.956418 Hz | AC cursor screenshot |
| Optimized fH | 4.060903 MHz | AC cursor screenshot |
| Optimized bandwidth | ≈4.060854 MHz | fH − fL |
| Output maximum | +193.97144 mV | Transient cursor |
| Output minimum | −200.94379 mV | Transient cursor |
| Output peak-to-peak | 394.91523 mV | Difference of cursor values |
| Input peak-to-peak | 20 mV | 10 mV peak source |
| Transient gain | ≈19.75 V/V | Vout(pp)/Vin(pp) |